PMT INTERFACE FOR YSI 9600.                   Revised:  February 24, 2005

Revision: B

Bill Of Materials             February   24, 2005     15:43:16     Page    1

Item  Quantity  Reference                 Part              Value      Tol       Rating    Type                                    Manufacturer        Part Number                   Supplier

_______________________________________________________________________________________________________________________________________________________________________________________________

1       2    C7                        0.01,,,           0.01 uF    +/-5%     16VDC     Ceramic, NP0 Dielectric, SMD 0805       AVX Corporation     0805YA103JAT2A               Digi-Key 800-344-4539

C8                        0.01,,,           0.01 uF    +/-5%     16VDC     Ceramic, NP0 Dielectric, SMD 0805       AVX Corporation     0805YA103JAT2A               Digi-Key 800-344-4539

2       3    C1                        0.1,,,            0.1 uF     +/-10%    25VDC     Ceramic, X7R Dielectric, SMD 0805       Panasonic           ECJ2VB1E104K                 Digi-Key 800-344-4539

C2                        0.1,,,            0.1 uF     +/-10%    25VDC     Ceramic, X7R Dielectric, SMD 0805       Panasonic           ECJ2VB1E104K                 Digi-Key 800-344-4539

C3                        0.1,,,            0.1 uF     +/-10%    25VDC     Ceramic, X7R Dielectric, SMD 0805       Panasonic           ECJ2VB1E104K                 Digi-Key 800-344-4539

3       1    C6                        100uF,,20V,TANT   100 uF     +/-20%    20VDC     Tantalum, Low ESR, Industrial, 7343-43  Kemet               T494X107M020AS               Digi-Key 800-344-4539

4       2    C4                        100uF,,10V,TANT   100 uF     +/-10%    10VDC     Tantalum, Low ESR, Industrial, 7343-31  Kemet               T494D107K010AS               Digi-Key 800-344-4539

C5                        100uF,,10V,TANT   100 uF     +/-10%    10VDC     Tantalum, Low ESR, Industrial, 7343-31  Kemet               T494D107K010AS               Digi-Key 800-344-4539

5       2    TP1                       PAD,,,            -          -         -         Test Point, 20AWG Metal Bus Wire                            -                            -

TP2                       PAD,,,            -          -         -         Test Point, 20AWG Metal Bus Wire                            -                            -

6       1    JP1                       JUMPER_1X2,,,     1 x 2      -         -         .25" Square Wrap Post .1" Center 2 Rows Samtec              TSW-136-07-G-D               Samtec   812-944-6733

7       1    R2                        51.1,,,AXIAL      51.1       5%        1/8W      Metal Film, +/-100 ppm TC, Mini Axial                       -                            Lab Stock

8       1    R3                        1k,,,AXIAL        1k         5%        1/8W      Metal Film, +/-100 ppm TC, Mini Axial                       -                            Lab Stock

9       1    R8                        100,,,            100        1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

10       2    R1                        470,,,            470        1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

R5                        470,,,            470        1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

11       1    R6                        1k,,,             1k         1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

12       1    R7                        10k,,,            10k        1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

13       1    R4                        100k,,,           100k       1%        1/8W      Metal Film, +/-100 ppm TC, SMD 0805                         -                            Lab Stock

14       2    D1                        1N6263,,,         1N6263     -         60V       Diode, Schottky, 15mA, 410mV, SOD-123   Diodes Inc          1N6263W-7                    Digi-Key 800-344-4539

D2                        1N6263,,,         1N6263     -         60V       Diode, Schottky, 15mA, 410mV, SOD-123   Diodes Inc          1N6263W-7                    Digi-Key 800-344-4539

15       1    U2                        74LV221A,,,       74LV221A   -         -         Dual Mono Multivibrator, SOIC           TI                  SN74LV221AD                  Digi-Key 800-344-4539

16       1    J2                        AMPMODU_7,,,HORZ  7 Pin      -         -         1 Row Right Angle Detent Latch Header   AMP                 103361-5                     Digi-Key 800-344-4539

17       1    J1                        AMPMODU_6,,,VERT  6 Pin      -         -         1 Row Vertical Detent Latch Header      AMP                 103414-4                     Digi-Key 800-344-4539

18       2    Q1                        2N7002,,,         2N7002     -         60V       N-Channel, Id .5A,  Rds 7.5 Ohm, SOT-23 Fairchild           2N7002NCT                    Digi-Key 800-344-4539

Q2                        2N7002,,,         2N7002     -         60V       N-Channel, Id .5A,  Rds 7.5 Ohm, SOT-23 Fairchild           2N7002NCT                    Digi-Key 800-344-4539

19       1    U3                        ADUM1201AR,,,     ADuM1201   -         1Mbps     Dual-Channel Digital Isolator, SOIC     Analog Devices      ADuM1201AR

20       1    U1                        NDL1205S,,,       5VDC       -         2W        Isolated DC/DC, Reg +5VDC Output, SIP   C&D Technologies    NDL1205S                     Mouser